# Observatory Manager Functional Requirements

# Connection

- A connect button must connect to all configured real devices.

- Each device must only have one connection regardless of how many times it is used in virtual properties.

# Virtual ObservingConditions device

- The virtual ObservingConditions device will aggregate up to 12 real Alpaca ObservingConditions devices into a single virtual device.

- Configuration options must be provided to enable the user to:

  - Create a list of available real ObservingConditions devices comprising:

    - devices selected from a list of discovered Alpaca devices

    - manually configured IP addresses of Alpaca devices

  - Associate each virtual device property with a real device property associated with one of the available real ObservingConditions devices.

# Virtual SafetyMonitor device

- Aggregate up to 12 Alpaca SafetyMonitor devices into a single virtual SafetyMonitor device.

- Enable each real device to be chosen from a list of discovered devices or by manual input of an IP address.

- Allow the user to enable or disable inclusion of each safety monitor in the aggregated list.

- The virtual IsSafe value must be determined as an OR of all enabled and connected safety monitor devices

- When IsSafe is true a rationale must be exposed through the Action/SupportedActions mechanic using the agreed JSON form.

- The conditions under which the safety monitor will return IsSafe = False must be determined by a set of user configurable rules

## Virtual SafetyMonitor Rules

Each real SafetyMonitor device will have these rules associated with it:

- A rule that includes or excludes the state of the sensor from consideration

- A rule that forces a given True / False state to be returned when the device is in an error state

The virtual device’s IsSafe property will return False when any real device’s IsSafe property is False, after it’s configured rules have been applied.

## ObservingConditions Rules

Each property in the virtual observing conditions device will have one or more rules associated with it. When the rule evaluates to True the virtual safety monitor IsSafe property will return False and when it evaluates to False the virtual safety monitor IsSafe property will return True.

The following rules will be associated with each virtual ObservingConditions device property:

- A rule that includes or excludes the state of the sensor from consideration

- A rule that returns IsSafe = False when:

  - The observed value exceeds a given threshold

  - The observed value is below a given threshold

  - The observed value equals a specified value

  - The device is in an error state

- A rule that forces a given True / False state to be returned when the device is in an error state